# Generate signals using PWM

Aim: To generate signals using PWM

Experimental requirements: PC loaded with Keil simulator and with required device pack

# Theory:

Pulse Width Modulation (PWM) is a widely used technique to generate variable output signals with a fixed frequency by controlling the width of the pulses. Pulse Width Modulation (PWM) is a technique used to generate an analog-like output using a digital signal by varying the width of the pulse while keeping the frequency constant. The duty cycle of the PWM signal determines the effective output voltage delivered to the load. In the LPC1769 microcontroller, PWM is generated using the PWM1 peripheral.

PWM is a digital signal where the duty cycle of the signal determines the amount of time the signal is high (ON) within each cycle. The frequency of the signal determines how many cycles occur in a second.

- Duty cycle: The proportion of time the signal is HIGH in one period.

- Period: The duration of one cycle of the signal (inverse of frequency).

- Frequency: The number of cycles per second (measured in Hz).

Applications: It’s used in various applications like motor control, signal modulation, dimming of LEDs,

and audio generation.

- Frequency (f): The rate at which the PWM signal completes one cycle (ON and OFF phases). This is usually measured in Hertz (Hz).

- Duty Cycle: The percentage of time the signal remains HIGH (ON) during one complete cycle. It’s

typically represented as a percentage (0% to 100%).

The PWM functionality in the LPC1769 is integrated with the Timer peripheral, which allows for precise

## Key Registers and Parameters:

The following key registers and parameters are involved in PWM generation on the LPC1769:

- Timer Control Register (TCR):

  - The TCR register is used to control the operation of the Timer. It has bits to enable the counter, reset the counter, and enable PWM mode.

    - TCR[0]: Enable Timer counting.

    - TCR[3]: Enable PWM mode.

- Match Registers (MR0, MR1, MR2, MR3):

  - The Match Registers (MR) hold values that determine the PWM period

  - (MR0) and the pulse width (MR1, MR2, etc.).

    - MR0: Defines the period of the PWM signal.

    - MR1, MR2, MR3: These match registers are used to control different PWM outputs.

- PWM Match Control (MCR):

  - This register is used to configure actions that occur when a match is made with the values in the match registers.

    - MCR[0]: Generate an interrupt when a match occurs.

    - MCR[1]: Reset the timer when a match occurs.

    - MCR[2]: Stop PWM when a match occurs.

- PWM Enable (PCR) Register:

  - The PWM Control Register (PCR) is used to enable PWM functionality on the specific pins.

## PWM Configuration Steps in LPC1769:

Here is the basic process for setting up PWM on P2.0 (PWM1.1) using the Timer1 module:

- Enable the PWM and Timer peripheral:

The first step is to power up the PWM1 and Timer1 peripherals. This is done by setting the appropriate bit in the PCONP (Power Control for Peripherals) register.

- LPC_SC->PCONP |= (1 << 6); // Power up PWM1 peripheral

- LPC_SC->PCONP |= (1 << 2); // Power up Timer1 peripheral

- Configure the PWM Pin:

Set the PINSEL register to select the PWM1.1 output (P2.0).

- LPC_PINCON->PINSEL4 &= ~(3 << 0); // Clear previous settings

- LPC_PINCON->PINSEL4 |= (1 << 0);  // Set P2.0 to PWM1.1 (output)

- Set PWM Period (MR0):

The period of the PWM signal (how long one cycle lasts) is set in MR0. For example, setting the period for a 1 kHz PWM signal:

- LPC_PWM1->MR0 = 25000; // Set PWM period to 1 kHz (assuming a 25 MHz system clock)

- Set PWM Duty Cycle (MR1):

The MR1 register controls the ON time (width of the pulse). For example, to set a 50% duty cycle:

- LPC_PWM1->MR1 = 12500; // Set PWM duty cycle to 50% (half of MR0)

- Enable PWM and Timer:

After setting the period and duty cycle, you need to enable the PWM output and the Timer.

- LPC_PWM1->TCR = (1 << 0) | (1 << 3); // Enable Timer and PWM mode

- Load the Match Registers:

The MR0 (period) and MR1 (pulse width) values are latched to activate the PWM signal.

- LPC_PWM1->LER = (1 << 0) | (1 << 1); // Latch the MR0 and MR1

- Start PWM Signal:

The PWM signal will start once the TCR[3] (enable PWM mode) bit is set.

On the LPC1769 microcontroller (from the LPC17xx family), PWM is provided via two peripherals:

| PWM Peripheral | Name | Channels (Outputs) | Counter Width | Mode Support |
| --- | --- | --- | --- | --- |
| PWM0 | PWM0.x | 6 channels (0.1–0.6) | 32-bit | Edge-aligned only |
| PWM1 | PWM1.x | 6 channels (1.1–1.6) | 32-bit | Edge & center-aligned |

## PWM0 Output Channels & Pins

| PWM0.x | Pin Name | Pin Location | PINSEL Reg | Bits | Function Value |
| --- | --- | --- | --- | --- | --- |
| 0.1 | PWM0.1 | P1.18 | PINSEL3 | 5:4 | 10 |
| 0.2 | PWM0.2 | P1.20 | PINSEL3 | 9:8 | 10 |
| 0.3 | PWM0.3 | P1.21 | PINSEL3 | 11:10 | 10 |
| 0.4 | PWM0.4 | P1.23 | PINSEL3 | 15:14 | 10 |
| 0.5 | PWM0.5 | P1.24 | PINSEL3 | 17:16 | 10 |
| 0.6 | PWM0.6 | P1.26 | PINSEL3 | 21:20 | 10 |

Note: For each of these, PINSEL3 must be set to 10 to select the PWM0 function.

## PWM1 Output Channels & Pins

| PWM1.x | Pin Name | Pin Location | PINSEL Reg | Bits | Function Value |
| --- | --- | --- | --- | --- | --- |
| 1.1 | PWM1.1 | P2.0 | PINSEL4 | 1:0 | 01 |
| 1.2 | PWM1.2 | P2.1 | PINSEL4 | 3:2 | 01 |
| 1.3 | PWM1.3 | P2.2 | PINSEL4 | 5:4 | 01 |
| 1.4 | PWM1.4 | P2.3 | PINSEL4 | 7:6 | 01 |
| 1.5 | PWM1.5 | P2.4 | PINSEL4 | 9:8 | 01 |
| 1.6 | PWM1.6 | P2.5 | PINSEL4 | 11:10 | 01 |

Note: For PWM1 output pins, set PINSEL4 to 01 for the correct bits to enable PWM1 function.

| Peripheral | Usable PWM Channels | Available Output Pins |
| --- | --- | --- |
| PWM0 | PWM0.1 to PWM0.6 | P1.18, P1.20, P1.21, P1.23, P1.24, P1.26 |
| PWM1 | PWM1.1 to PWM1.6 | P2.0 to P2.5 |

## Algorithm:

- Start the program.

- Enable power to PWM1 peripheral

  - Set the corresponding bit in the PCONP register.

- Configure the PWM output pin

  - Select pin P2.0 as PWM1.1 function using the PINSEL register.

- Set the PWM prescaler value

  - Configure the prescale register to zero so the PWM runs at the system clock frequency.

- Set the PWM time period

  - Load Match Register MR0 with the value corresponding to the required PWM frequency (1 kHz).

- Set the PWM duty cycle

  - Load Match Register MR1 with a value equal to 50% of MR0 to generate a 50% duty cycle.

- Load the match register values

  - Enable the latch for MR0 and MR1 so the new values take effect.

- Enable PWM output

  - Configure the PWM Control Register to enable PWM1 output.

- Start the PWM timer

  - Reset and enable the PWM counter using the Timer Control Register.

- Enter infinite loop

  - Keep the PWM running continuously.

- End.

# Program Code:

#include<LPC17xx.h> void PWM_init(void){

LPC_SC->PCONP |=(1<<6);	//power on LPC_SC->PCLKSEL0 |=(1<<13);		//pwm clk

LPC_PINCON->PINSEL4 |=(1<<0);				//pin selection LPC_GPIO2->FIODIR |=(1<<0);			// 2.0 pwm output pin LPC_PWM1->PR=9;	// PR VALUE LPC_PWM1->MR0=1000;		// MR VALUE

LPC_PWM1->MR1=250;	//MR1 VALUE

LPC_PWM1->MCR=(1<<0)|(1<<1);		// UPDATED THE MR & MR1 VALUES LPC_PWM1->PCR =(1<<9);	// SINGLE EDGE ENABLE

LPC_PWM1->LER=(1<<0)|(1<<1);	// UPDATE THE MR0 ADN MR1 VALUES LPC_PWM1->TCR=(1<<0)|(1<<3);	 // TIMER START AND ENABLE	}

int main (void){	// MAIN FUNCTION

PWM_init();	// CALL THE FUNCTION LPC_PWM1->TCR=1;		// START THE TIMER

## Input:

while(1){

}}

- System clock frequency applied to the LPC1769 microcontroller.

- PWM period value loaded in Match Register MR0 = 1000.

- PWM duty cycle value loaded in Match Register MR1 = 500.

- PWM output pin selected as P2.0 (PWM1.1).

## Output:

Precautions:

- Ensure the correct LPC1769 device is selected in the Keil project settings before compiling.

- Include the correct LPC17xx header file to avoid register definition errors.

- Enable the PWM peripheral clock before accessing any PWM registers.

- Configure the PINSEL register correctly to assign the pin to PWM function instead of GPIO.

- Verify the PWM clock frequency and prescaler value to obtain the desired output frequency.

- Load the match register values (MR0 and MR1) properly and enable the latch register to update them.

- Ensure the duty cycle value is less than or equal to the period value (MR1 ≤ MR0).

- Enable the PWM output using the PWM Control Register (PCR) before starting the timer.

- Reset and start the PWM counter correctly using the Timer Control Register (TCR).

- Check for compilation errors and warnings before running the simulation.

- Use the Logic Analyzer / Peripheral View in Keil to observe the PWM waveform accurately.

- Do not modify PWM registers while the counter is running unless the latch mechanism is used.

## Result:

A PWM signal of 1 kHz frequency with 50% duty cycle is successfully generated at pin P2.0 (PWM1.1) and observed in the Keil simulator.