# Microprocessor and Microcontroller   Case Studies

# 1. Matrix Keyboard Interfacing and Input Detection Using TM4C123 Microcontroller

## Abstract

A matrix keypad is commonly used in embedded systems to provide user input while minimizing the number of I/O pins required. In this experiment, a 4×4 matrix keyboard is interfaced with the TM4C123 microcontroller. The microcontroller scans rows and columns to detect which key is pressed and processes the detected input.

## Aim

To interface a 4×4 matrix keypad with the TM4C123 microcontroller and detect key presses using a row-column scanning method.

## Objectives

1. To understand matrix keypad interfacing with a microcontroller.

2. To learn the row-column scanning technique.

3. To configure GPIO ports of the TM4C123 microcontroller.

4. To detect and process keypad inputs.

## Experimental Requirements

Hardware:

• TM4C123 microcontroller development board

• 4×4 Matrix Keypad

• Breadboard and connecting wires

• Power supply

Software:

• Keil µVision / Code Composer Studio

• Embedded C compiler

## Theory

A matrix keypad consists of rows and columns arranged in a grid format. Each key is located at the intersection of a row and a column. The microcontroller sets rows as outputs and columns as inputs. When a key is pressed, the row and column become electrically connected, allowing the microcontroller to detect which key is pressed by scanning the rows and reading columns.

## Procedure

1. Connect keypad rows to GPIO output pins of TM4C123.

2. Connect keypad columns to GPIO input pins with pull-up resistors.

3. Initialize the GPIO ports.

4. Activate one row at a time.

5. Read the column inputs.

6. Identify the pressed key based on row and column detection.

## Program

#include <stdint.h>

#include "tm4c123gh6pm.h"

int main()

{

SYSCTL_RCGCGPIO_R |= 0x02;

GPIO_PORTB_DIR_R = 0x0F;

GPIO_PORTB_DEN_R = 0xFF;

while(1)

{

GPIO_PORTB_DATA_R = 0x01;

if((GPIO_PORTB_DATA_R & 0x10) == 0)

{

// Key detected

}

}

}

## Observations

| ## Row Activated | ## Column Detected | ## Key Pressed | ## Output on Terminal/LCD |
| --- | --- | --- | --- |
| ## Row 1 | ## Column 1 | ## 1 | ## 1 |
| ## Row 1 | ## Column 2 | ## 2 | ## 2 |
| ## Row 1 | ## Column 3 | ## 3 | ## 3 |
| ## Row 1 | ## Column 4 | ## A | ## A |
| ## Row 2 | ## Column 1 | ## 4 | ## 4 |
| ## Row 2 | ## Column 2 | ## 5 | ## 5 |
| ## Row 2 | ## Column 3 | ## 6 | ## 6 |
| ## Row 2 | ## Column 4 | ## B | ## B |
| ## Row 3 | ## Column 1 | ## 7 | ## 7 |
| ## Row 3 | ## Column 2 | ## 8 | ## 8 |

## Precautions

• Ensure correct GPIO configuration.

• Use pull-up resistors for input pins.

• Avoid pressing multiple keys simultaneously.

## Applications

• ATM keypads

• Security systems

• Industrial control panels

## Result

Matrix keypad interfacing with TM4C123 was successfully implemented and key presses were detected.

# 2. Generation of Variable Duty Cycle PWM Signals Using TM4C123 Microcontroller Timer Module

## Abstract

Pulse Width Modulation (PWM) is used to control power delivered to electronic devices. In this experiment, the timer module of the TM4C123 microcontroller is used to generate PWM signals with variable duty cycles.

## Aim

To generate PWM signals with variable duty cycles using the timer module of the TM4C123 microcontroller.

## Objectives

1. To understand PWM signal generation.

2. To configure timer modules for PWM.

3. To vary duty cycle programmatically.

4. To observe PWM waveform.

## Experimental Requirements

Hardware:

• TM4C123 microcontroller board

• LED or DC Motor

• Oscilloscope

• Resistors

Software:

• Code Composer Studio / Keil µVision

## Theory

PWM controls average voltage by varying ON and OFF times of a digital signal. The duty cycle determines how long the signal stays HIGH within one period.

## Procedure

1. Enable timer module clock.

2. Configure timer in PWM mode.

3. Set PWM period register.

4. Set duty cycle value.

5. Enable PWM output and observe waveform.

## Program

#include <stdint.h>

#include "tm4c123gh6pm.h"

int main()

{

SYSCTL_RCGCTIMER_R |= 0x01;

TIMER0_CTL_R = 0x00;

TIMER0_CFG_R = 0x04;

TIMER0_TAMR_R = 0x0A;

TIMER0_TAILR_R = 16000;

TIMER0_TAMATCHR_R = 8000;

TIMER0_CTL_R |= 0x01;

while(1)

{

}

}

## Observations

| PWM Register Value | Duty Cycle (%) | ON Time | OFF Time | Output Observation |
| --- | --- | --- | --- | --- |
| 4000 | 25% | Short | Long | Dim LED / Slow motor |
| 8000 | 50% | Equal | Equal | Medium LED brightness |
| 12000 | 75% | Long | Short | Bright LED / Faster motor |
| 16000 | 100% | Continuous ON | 0 | LED fully ON |

## Precautions

• Configure timer registers correctly.

• Ensure proper PWM pin configuration.

• Verify waveform using oscilloscope.

## Applications

• Motor speed control

• LED brightness control

• Servo motor control

## Result

Variable duty cycle PWM signals were successfully generated using the TM4C123 timer module.